C***BEGIN DESCRIPTION GLOBALS

C

C   This file is a description og the global variables

C   used in connection with the Princeton Ocean Model (POM).

C

C   The variables are grouped according to their type and dimension.

C

C***DATE WRITTEN   910626 (YYMMDD)

C***REVISION DATE  911122 (YYMMDD)

C***AUTHOR

C            Jarle Berntsen, Institute of Marine Research,

C            Postboks 1870,

C            N-5024 Bergen-Nordnes, Norway

C            Email..  jarleb@imr.no

C

C***INTEGERS

C

C   Integers defining the grid size.

C

C   IM     Number of cells in X-direction.

C   JM     Number of cells in Y-direction.

C   KB     Number of sigma layers + 1.

C   LIJ    IM*JM.

C

C   Integers defining the model time or time periods.

C

C   NUMSTP Internal mode time index.

C   IPRTD1 Long interval in hours between printouts.

C   IPRTDT Short interval in hours between printouts.

C   IPRINT Long interval in NUMSTP between printouts.

C   IPRTOT Short interval in NUMSTP between printouts.

C   IDAYS  The number of days the model is run.

C   ISPLIT The number of external mode steps for each internal mode

C          step.

C   ISPADV The number of external mode steps between each time some

C          vertical integrals are updated.

C   NDIVIS The number of internal steps per hour.

C   IEND   The number of internal step in IDAYS days.

C   NATMO  Maximum allowed number of hours between new atmospheric data.

C   IDIFH  Time difference in hours between atmospheric data.

C   INTMET When INTMET = NUMSTP, we read new atmospheric forcing.

C

C   Integers defining the size of the FRS-zones.

C

C   LB     The number of cells in the FRS-zone.

C   MC     LB+1.

C   MM10   IM-LB+1.

C   NM10   JM-LB+1.

C

C   Integers connected to data files.

C

C   NNIN   File unit number of the bottom topography data file.

C   NYIN   File unit number of the atmospheric data file.

C   NDSI   File unit number of the output file.

C   NUNNIN Record number of the first record to be read when we read

C          from NNIN with the routine RFLT.

C   NUNYIN Record number of the first record to be read when we read

C          from NYIN with the routine RFLT.

C   NUNDSI Record number of the previous record when writing

C          to NNDSI with the routine PUTFLT.

C   LENREC Record length of direct access input and output files.

C   MAXDIM Dimension of the array BBGRD used as a working array when

C          reading atmospheric data.

C   LINN   The number of z-levels of input data to the model.

C          The z-level data is interpolated to sigma layers by

C          the routine Z2SIGMA.

C

C   Administration integers.

C

C   MODE   If MODE = 2, the model is run in barotropic 2-D mode.

C          If MODE = 3, the model is run prognostic 3-D. That is:

C                       salinity and temperature are advected.

C          If MODE = 4, the model is run diagnostic 3-D. That is:

C                       without advecting salinity and temperature.

C   MEDO   Is used when calling PTXY and NEWPTXY.

C          MEDO = 0 on the first call.

C          MEDO = 2 on later calls.

C

C   1-D INTEGER arrays.

C

C   NDATE  Integer array of dimension 5.

C          NDATE(1) defines the year.

C          NDATE(2) defines the month.

C          NDATE(3) defines the day.

C          NDATE(4) defines the hour.

C          NDATE(5) is the number of hours since the start of the

C          computation.

C   IDENTI Integer array of dimension 20.

C          This is the DNMI filed head identifying 2-D gridded arrays

C          of model results.

C          IDENTI(1) identifies the producer.

C          IDENTI(2) grid area.

C          IDENTI(3) data type.

C          IDENTI(4) time parameter.

C          IDENTI(5) vertical coordinate.

C                    IDENTI(5) = 1 for z-level data.

C                    IDENTI(5) = 2 for sigma-level data.

C          IDENTI(6) identifies the parameter.

C                    IDENTI(6) = 301 for water elevation.

C                    IDENTI(6) = 302 for U-comp. of velocity.

C                    IDENTI(6) = 303 for V-comp. of velocity.

C                    IDENTI(6) = 307 for salinity.

C                    IDENTI(6) = 308 for temperature.

C                    IDENTI(6) = 351 for bottom topography.

C          IDENTI(7) level 1?

C          IDENTI(8) level 2?

C          IDENTI(9) classifies the grid.

C                    IDENTI(9) = 1 for rectangular grids.

C          IDENTI(10) = IM.

C          IDENTI(11) = JM.

C          IDENTI(12) = year.

C          IDENTI(13) = month*100 + day.

C          IDENTI(14) = hour*100 + minute.

C          IDENTI(15) = XP*100.

C          IDENTI(16) = YP*100.

C                   XP and YP are the x-coordinate and the y-coordinate

C                   respectively of the North Pole in the given grid.

C          IDENTI(17) = (grid spacing in KM) * 10.

C          IDENTI(18) = Angle of the Y-axis if the grid.

C          IDENTI(19) is avaliable for different purposes.

C                     IDENTI(19) = sigma*10000 when

C                     IDENTI(5) = 2 (sigma coordinate data).

C          IDENTI(20) defines the scaling of the parameters.

C                     real value = stored value * 10**(IDENTI(20)).

C

C   IOUTGRID Integer array of dimension 4.

C            Is defined in DYPGRIN. (trenger vi denne?)

C            IOUTGRID(I) = IDENTI(14+I),I=1,2,3 and 4.

C   ISTART Integer array of dimension 4.

C          Defines the start time of the computation.

C          ISTART(1) = year.

C          ISTART(2) = month.

C          ISTART(3) = day.

C          ISTART(4) = hour.

C   IDATEMET Integer array of dimension 4.

C          Defines the time for reading atmospheric forcing.

C          trenger vi denne?

C

C***REALS

C

C   DTI    The internal time step. (s)

C   DTE    The external time step. (s)

C   DTI2   2*DTI. (s)

C   DTE2   2*DTE. (s)

C   UMOL   The minimum allowed value of the vertical diffusion

C          coefficient. (m**2/s)

C   GRAV   Gravity. (m/s**2)

C   RHO0   Reference density of sea water. (kg/m**3)

C   ROAIR  Reference density of air. (kg/m**3)

C   DX     Grid spacing in x-direction. (m)

C   DY     Grid spacing in y-direction. (m)

C   DEPMAX Maximum depth. (m)

C   HORCON Horisontal diffusion constant used in connection

C          with Smagorinsky's technique. (dimensionless)

C   SMOTH  Smoothing parameter used to remove computational

C          modes from the leapfrog approximations. (dimensionless)

C   SMALL  A small parameter used to avoid division by zero

C          when computing the vertical turbulence.

C   PMEAN  Average atmospheric pressure. (Pa = N/m**2 = kg/(m*s**2) )

C   DIVROG = 1/(RHO0*GRAV).

C   CFLCRI The Courant-Friedrichs-Lewy criterion for stability. (s)

C

C***1-D REAL ARRAYS

C

C   Arrays of dimension KB.

C

C   Z      Sigma coordinate interfaces between layers,

C          Z(1) = 0. and Z(KB) = -1.

C   DZ     Thickness of the sigma-layers.(DZ(K)=Z(K)-Z(K+1))

C   ZZ     The sigma coordinate depths of the (S,T) points.

C          (ZZ(K) = (Z(K)+Z(K+1))/2).

C   DZZ    Vertical sigma-coordinate distance between (S,T) points.

C          (DZZ(K) = ZZ(K) - ZZ(K+1)).

C   DZR    Inverse of the sigma coordinate thickness of the layers.

C          (DZR(K) = 1/DZ(K)).

C

C          Z,DZ,ZZ,DZZ and DZR are dimensionless.

C

C   BBGRD  Array of dimension MAXDIM.

C          BBGRD is used as a working array when reading

C          atmospheric data.

C   GRIDAA Array of dimension 4. GRIDAA contains neccesary

C          grid information to be used when transforming

C          the atmospheric forcing on to our grid.

C

C***2-D REAL ARRAYS

C

C   Arrays of dimension (IM,JM)

C

C   H      The depths. (m)

C   EL     The surface elevation as used in the external mode. (m)

C   ELF    The external mode surface elevation at time +DTE. (m)

C   ELB    The external mode surface elevation at time -DTE. (m)

C   ET     The surface elevation as used in the internal mode. (m)

C   ETF    The internal mode surface elevation at time +DTI. (m)

C   ETB    The internal mode surface elevation at time -DTI. (m)

C   EGF    The average of the external mode surface elevations

C          over the time (time,time+DTI). (m)

C   EGB    The average of the external mode surface elevations

C          over the time (time-DTI,time). (m)

C   D      Dynamic depth, H + EL. (m)

C   DT     Dynamic depth, H + ET. (m) (Uheldig valg av navn?)

C   FSM    Mask for scalar variables. FSM = 0 over land cells,

C          and FSM = 1 over sea cells.

C   DUM    Mask for the U-component of the velocity.

C          DUM = 0 over land and 1 over sea.

C   DVM    Mask for the V-component of the velocity.

C          DVM = 0 over land and 1 over sea.

C   UA     Depth integrated value of U. (m/s)

C   UAF    Depth integrated value of U at time +DTE. (m/s)

C   UAB    Depth integrated value of U at time -DTE. (m/s)

C   UTF    The average of UA over the time (time,time+DTI). (m/s)

C   UTB    The average of UA over the time (time-DTI,time). (m/s)

C   VA     Depth integrated value of V. (m/s)

C   VAF    Depth integrated value of V at time +DTE. (m/s)

C   VAB    Depth integrated value of V at time -DTE. (m/s)

C   VTF    The average of VA over the time (time,time+DTI). (m/s)

C   VTB    The average of VA over the time (time-DTI,time). (m/s)

C   CBC    Bottom friction coefficient. (dimensionless)

C   COR4   Coriolis parameter/4. (1/s)

C   WUSURF Momentum flux in X-direction at the surface. (m**2/s**2)

C   WVSURF Momentum flux in Y-direction at the surface. (m**2/s**2)

C   DUSURF Difference in WUSURF from one atmospheric forcing

C          to the next. (m**2/s**2)

C   DVSURF Difference in WVSURF from one atmospheric forcing

C          to the next. (m**2/s**2)

C   WTSURF Surface boundary condition for temperature. (Celcius)

C   WSSURF Surface boundary condition for salinity. (Practical Salinity)

C   WUBOT  Momentum flux in X-direction at the bottom. (m**2/s**2)

C   WVBOT  Momentum flux in Y-direction at the bottom. (m**2/s**2)

C   AAM2D  Horizontal depth integrated diffusivity. (m**2/s)

C   PATM   Atmospheric pressure. (Pa = N/m**2 = kg/(m*s**2) )

C   DATM   Difference in atmospheric pressure. (Pa)

C   ADVUA  Sum of the advection and diffusion terms of the

C          vertical integrated moment equation in U times the

C          area of the cell. (m**4/s**2).

C   ADVVA  Same as ADVUA, but for the moment equation in V.

C   ADVUU  Sum of advection and diffusion terms of the

C          vertical integrated moment equation in U times the

C          area of the cell. (m**4/s**2).

C   ADVVV  Same as ADVUU, but for the moment equation in V.

C          ADVUU and ADVVV are modified for each internal step.

C          ADVUA and ADVVA are modified each ISPADV'th external step.

C   TRNU   Depth integrated values of two vertical integrals in the

C          moment equation in U times the are of the cell. (m**4/s**2)

C   TRNV   Same as TRNU, but for the moment equation in V.

C

C   FLUXUA Working array.

C   FLUXVA Working array.

C   TPS    Working array.

C   C11    Working array.

C   C12    Working array.

C   C21    Working array.

C   C22    Working array.

C   DENOM  Working array.

C   VELOUT Working array.

C   FW     Working array.

C

C   The following variables:

C          H,EL,ELF,ELB,ET,ETF,ETB,EGF,EGB,D,DT,FSM,CBC,COR4,

C          WUSURF,WVSURF,DUSURF,DVSURF,WTSURF,WSSURF,AAM2D,PATM,

C   are defined in the (S,T)-points of the 2-D Arakawa C-grid.

C

C   The following variables:

C          DUM,UA,UAF,UAB,WUBOT,ADVUA, ADVUU,TRNU,

C   are defined in the U-points of the 2-D Arakawa C-grid.

C

C   The following variables:

C          DVM,VA,VAF,VAB,WVBOT,ADVVA,ADVVV,TRNV,

C   are defined in the V-points of the 2-D Arakawa C-grid.

C

C***3-D REAL ARRAYS

C

C   Arrays of dimension (IM,JM,KB).

C

C   U      Horisontal velocity in X-direction

C          at timestep n. (m/s)

C   UF     Horisontal velocity in X-direction

C          at timestep n + DTI. (m/s)

C   UB     Horisontal velocity in X-direction

C          at timestep n - DTI. (m/s)

C   V      Horisontal velocity in Y-direction

C          at timestep n. (m/s)

C   VF     Horisontal velocity in Y-direction

C          at timestep n + DTI. (m/s)

C   VB     Horisontal velocity in Y-direction

C          at timestep n - DTI. (m/s)

C   W      Sigma coordinate vertical velocity. (m/s)

C   WR     Physical coordinate vertical velocity. (m/s)

C

C   T      Temperature at timestep n. (Celcius)

C   TB     Temperature at timestep n - DTI. (Celcius)

C   TMEAN  Mean temperature. (Celcius)

C   S      Salinity at timestep n. (Practical Salinity)

C   SB     Salinity at timestep n - DTI. (Practical Salinity)

C   SMEAN  Mean salinity. (Practical Salinity)

C   RHO    Density. (kg/m**3)

C   RMEAN  Mean density. (kg/m**3)

C

C   Q2     Turbulence kinetic energy times two

C          at timestep n. (m**2/s**2)

C   Q2B    Turbulence kinetic energy times two

C          at timestep n - DTI. (m**2/s**2)

C   Q2L    Q2 * Turbulence length scale at timestep n. (m**3/s**2)

C   Q2LB   Q2 * Turbulence length scale at timestep n - DTI.

C   KM     Vertical eddy viscosity. (m**2/s)

C   KH     Vertical diffusion. (m**2/s)

C   KQ     Vertical viscosity used in the turbulence closure

C          scheme. (m**2/s)

C   L      Turbulence length scale. (m)

C

C   AAM    Horisontal diffusivities. (m**2/s)

C

C   DRHOX  X-Component of the internal baroclinic pressure

C          gradient. (m**4/s**2)

C   DRHOY  Y-Component of the internal baroclinic pressure

C          gradient. (m**4/s**2)

C

C   A      Working array.

C   C      Working array.

C   VH     Working array.

C   VHP    Working array.

C   PROD   Working array.

C   DTEF   Working array.

C   TCLIM1 Working array.

C          Climatological temperature at the 15th of month 1.

C   TCLIM2 Working array.

C          Climatological temperature at the 15th of month 2.

C   SCLIM1 Working array.

C          Climatological salinity at the 15th of month 1.

C   SCLIM2 Working array.

C          Climatological salinity at the 15th of month 2.

C

C   The following variables:

C       WR,T,TB,TMEAN,S,SB,SMEAN,RHO,RMEAN,AAM,

C   are defined in the (S,T)-points of the 3-D Arakawa C-grid.

C

C   The following variables:

C       U,UF,UB,DRHOX,

C   are defined in the U-points of the 3-D Arakawa C-grid.

C

C   The following variables:

C       V,VF,VB,DRHOY,

C   are defined in the V-points of the 3-D Arakawa C-grid.

C

C   The following variables:

C       W,Q2,Q2B,Q2L,Q2LB,KM,KH,KQ,L,

C   are defined in the W-points of the 3-D Arakawa C-grid.

C

C***END DESCRIPTION